# Informatics

# C Programming Assignment - algorithm, development notes and reflections.

### Tristan Yau 98080106.	Tutor: Craig Scott.

## Algorithm and Explanations

### Top Level Design.

The problem was to develop a program which:

- accepts a resistance from the user.

- outputs R1 and R2, total resistance and series or parallel connection, such that the result is the best E12 pairing (closest in resistance to Rinput).

### Second Level.

In the first version of the program:

  - accepts a resistance from the user.

  - checks input is within range. (1 £ input < 10 million).

  - formats the input to a floating point number less than 10.

  - outputs number and multiplier to main, which is then passed to the calculation function.

- initialise E12 series in an array.  E12 values are in the decade less than 10 (ie, 1, 1.2, ..... 8.2).

- calculate RT in series with resistors less than the Rinput, since adding up resistors in series with resistors greater than Rinput will not give the value of Rinput, but values up to twice Rinput.		RT=RA+RB

- calculate RT in parallel with resistors greater than Rinput, because using resistors less than Rinput will result in RT being up to half of Rinput.    RT=RARBRA+RB

- search the array in which the calculated values are stored, to find the combination that gives the lowest difference to Rinput.

- display answer.

|  | RA (E12 Series Resistors) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB | 1 | 1.2 | 1.5 | 1.8 | 2.2 | 2.7 | 3.3 | 3.9 | 4.7 | 5.6 | 6.8 | 8.2 |
| 1 | 2 | 2.2 | 2.5 | 2.8 | 3.2 | 3.7 |  |  |  |  |  |  |
| 1.2 |  | 2.4 | 2.7 | 3 | 3.4 | 3.9 |  |  |  |  |  |  |
| 1.5 |  |  | 3 | 3.3 | 3.7 | 4.2 |  |  |  |  |  |  |
| 1.8 |  |  |  | 3.6 | 4 | 4.5 |  |  |  |  |  |  |
| 2.2 |  |  |  |  | 4.4 | 4.9 |  |  |  |  |  |  |
| 2.7 |  |  |  |  |  | 5.4 |  |  |  |  |  |  |
| 3.3 |  |  |  |  |  |  | 1.65 | 1.79 | 1.94 | 2.08 | 2.22 | 2.35 |
| 3.9 |  |  |  |  |  |  |  | 1.95 | 2.13 | 2.3 | 2.48 | 2.64 |
| 4.7 |  |  |  |  |  |  |  |  | 2.35 | 2.56 | 2.78 | 2.99 |
| 5.6 |  |  |  |  |  |  |  |  |  | 2.8 | 3.07 | 3.33 |
| 6.8 |  |  |  |  |  |  |  |  |  |  | 3.4 | 3.72 |
| 8.2 |  |  |  |  |  |  |  |  |  |  |  | 4.1 |

The above array is an example of the values generated when Rinput is a multiple of 3 (eg, 30, 300, etc).  In this case, the best combination is 1.5 and 1.5 in series, or 1.2 and 1.8 in series.  If there are two exact answers like this, it does not matter which combination is reported, since we are looking for the best combination.
Note that the values for the blank array positions in the top right-hand corner (rectangle shape) are not generated because they are not acceptable (eg, 1 || 8.2 gives approximately 1).  The blank area (triangle shape) in the bottom left-hand side is exactly the same as the area on the other side of the y = x diagonal, so these values are not generated.

The problem with this algorithm is that it can only output answers from one decade of the E12 series.  What if the best answer was found using E12 resistors across different decades (eg, 10 + 220 ohms)?

In Version 2 of the program:

  - accepts a resistance from the user.

  - checks input is within range. (1 <= input < 10 million).

  - outputs the number to main, which then passes it to the calculation function.

- initialise E12 series in an array.  E12 values are in all the decades less than 10 million  (ie, 1, 1.2, ....., 8.2,  10, 12, ..... , 82, ..... , 8.2 million).

- as with the last version, do the same calculations for RT in series and RT in parallel.  Check if this value is closer to the desired value.  If so, store value, and the resistor combination.

- display the best combination stored in the variables.

Thus, the need for the array was eliminated, since the searching routine was combined with the calculation routines.  Also, because of the fact that all the E12 series’ needed are stored in the array, formatting the userÕs input (to a multiple less than 10) was not necessary.

Because of the lengthy time delay (on my slow computer), each search had to be restricted to the two consecutive decades higher and lower than the user’s input.  This degree of restriction was also chosen to reflect the user’s probable degree of accuracy required.  Using a 1K and a 1 ohm resistor is not practical, since the error with this combination is 0.1%, while the tolerance on the 1K ohm resistor is likely to be around 5% (ie, the 1K resistor was likely to vary by ±50 ohms anyway).

Due to the above improvements, Version 2 still does the same function (and did more calculations), but with fewer lines of code, compared with the original program.

## Development Notes

Programming Commenced:		Saturday 23rd May 1998.

Original Program Finished:		Tuesday 26th May 1998.

Version 2 Finished:			Sunday 31st May 1998.

Compiler Used:		gcc v2.8, gnu cpp v2.8 (2/2/98).  (downloaded from 					www.delorie.com).

OS:		The compiled program is fully functional in MS-DOS.  The code also works in UNIX, but the boxes and the omega symbol are not displayed correctly (since the UNIX ASCII character set does not have the required MS-DOS ASCII characters).

For more development notes, see comments in the code.

## Journal/Reflections

- My initial reaction to the problem was one of being overwhelmed - I had no idea at how to attack the problem.  Getting the idea of storing the generated values in an array came from the other 20% programming assignment (parallel resistor look up table).  I also had a look at the javascript code in a web page that did just about the same function as my problem.  I got the idea of formatting the input value into a number less than 10 from this example.  Also, some hints about the repetitive structure of the array, and how to make the code more robust, were given by my tutor, Craig Scott.

- Given the above examples and advice, I started to piece together a second level design.  Some of the detail was filled in later, when I was programming.

- Following the basic structure of the second level design, a program was developed, with corresponding functions for each step.

- Each function was developed in a separate file.  Firstly, the code was typed in.  Secondly, it was debugged.  Once bug-free, the function in that file was ‘plugged’ (cut and pasted) into the central file, and connected to the main() function.  This follows the ‘black box’ idea, where each function is a module that only does one thing.  This method of programming allows for easy debugging, as the function is relatively small compared with the whole program (less code to check).  However, this does not eliminate the errors that can occur in the main function, or in the operations between the various function calls.

- After one weekend of hard work (and persistence!), the basic program was up and running.  I re-evaluated the assignment task, to find if there were any fundamental faults in the ability of the program to satisfy the aim.  There was one.

- The design error was that the user’s input was formatted into exponential notation, to compare it with two E12 resistors in the decade from 1 to 10.  It occurred to me that there may be a better combination with two resistors across two consecutive decades.  This was overlooked at the beginning, since I started with the array of twelve E12 values, and built the program on that idea.

- The second algorithm was developed, with some other improvements, such as the elimination of the array of calculated values, and the reformatting of the output.  After a second gruelling weekend, Version 2 was complete.

- As they say, the rest is history!

### Other notes:

- The code that adds robustness to the program is always ten times the amount of the basic program itself!

- Testing the program is also time consuming.  Often, a bug may not appear until after the final compilation.

- The development time to fully code and test the program was a quite long, considering how easy it is just to work out the answer manually using another model such as a spreadsheet.

- However, I still see the value of this experience, for furthering our skills in program development (eg, meeting the clients’ needs), coding techniques (different algorithms) and problem solving using the tools we have in C.